# SentryRailTrack (SRT): Autonomous AI-Powered Train Condition Monitoring Platform

## 1. Executive Summary

Modern urban rail networks demand near-zero downtime and uncompromising safety standards. Singapore’s MRT system handles millions of passenger trips daily, making the prevention of unexpected component failures and peak-hour train withdrawals paramount. Traditional railway maintenance relies heavily on manual walk-through inspections during short non-revenue engineering windows (1:30 AM – 4:30 AM) and static, fixed-threshold SCADA alarms that trigger only after a failure occurs.

SentryRailTrack (SRT) bridges this gap by offering a hybrid edge-and-cloud autonomous condition monitoring system. Designed specifically for Problem Statement 3: Train Condition Monitoring, SRT pairs a custom-designed, track-mounted autonomous inspection robot (Edge Hardware) with a non-technical central diagnostic web suite (Cloud Platform) powered by specialized machine learning models and Generative AI (Google Gemini API via Google AI Studio). The system provides continuous, automated anomaly detection across four primary rolling stock and track subsystems: Door, Air-Conditioning & Ventilation (ACV), Rail Corrugation, and Structural Health Monitoring (SHM).

## 2. Problem Statement & Operational Inefficiencies

### 2.1 The Core Challenge

Singapore’s Land Transport Authority (LTA) and rail operators face significant operational bottlenecks due to legacy inspection paradigms:

- Coverage Gaps in Short Engineering Windows: Physical track sweeps can only inspect a small fraction of the total network per night. Degradation occurring between inspections remains undetected until the next cycle.

- Fixed-Threshold Blind Spots: SCADA systems monitor current, temperature, and pressure against static upper/lower limits. Subtle progressive degradation—such as micro-corrugation, elevated door motor friction, or slow refrigerant leaks—operates within normal bounds right up until catastrophic failure during peak-hour service.

- Data Silos & High Mean-Time-To-Detect (MTTD): Raw telemetry logs (.csv, .txt, .xlsx) captured across train cars and trackside sensors remain disconnected. When incidents occur, maintenance crews spend valuable time manually sifting through raw files to reconstruct fault sequences.

### 2.2 Real-World Manifestations of Component Wear

- Door Subsystem: Dust accumulation, dry grease, or misaligned guide rollers increase mechanical friction. The door still closes within timeout limits, but elevated motor current draw leads to silent motor overheating and sudden door jams during passenger boarding.

- ACV Subsystem: Slow refrigerant micro-leaks in a single carriage reduce cooling capacity without breaching hard temperature trip points during cool mornings, leading to total cooling failure during peak afternoon heat.

- Rail Corrugation: Early-stage microscopic surface wear along rail heads increases wheel-rail dynamic vibration, accelerating wheel wear and creating cabin noise before visual inspections can spot it.

- Structural Health Monitoring (SHM): Micro-fissures and dynamic stress on bogie frames and track beds accumulate silently over millions of vibration cycles, risking structural fatigue failures without visible warning signs.

## 3. System Architecture & End-to-End Workflow

[ SentryRailTrack (SRT) Edge Robot ] ───(Raw Telemetry)───► [ Central Diagnostic Web Suite ]
      (Multi-Modal Edge Sensing)                              (Vercel Serverless Python Engine)
                                                                           │
               ┌───────────────────┬───────────────────┬───────────────────┴────────┐
               ▼                   ▼                   ▼                            ▼
           [ Door ]              [ ACV ]       [ Rail Corrugation ]              [ SHM ]
        1D-CNN + BiLSTM         CatBoost            LightGBM                    Regression
      Temporal Segmentation  Leak Localization  Vibration Classification       Fatigue Regression
               │                   │                   │                            │
               └───────────────────┴─────────┬─────────┴────────────────────────────┘
                                             ▼
                                [ Automated Evaluation Output ]
                                     `predictions.zip`
                                             │
                                             ▼
                               [ Gemini LLM Diagnostic Layer ]
                            (Step-by-step SOPs via SRT Mobile App)

## 4. Subsystem Machine Learning Frameworks & Mathematical Formulations

Each subsystem uses a custom machine learning architecture built to comply directly with competition evaluation metrics:

### 4.1 Door Subsystem (Temporal Cycle Segmentation & Classification)

- Objective: Automatically segment opening/closing cycles from continuous time-series and classify them as Normal or Abnormal resistance.

- Algorithms: 1D-CNN + Bidirectional LSTM (BiLSTM).

- Mathematical Pipeline:

  - Signal Preprocessing: Z-score normalization across motor voltage , motor current , and back-EMF:

  - Loss Function: Optimized using Binary Cross-Entropy (BCE) combined with temporal Intersection over Union (IoU):

- Evaluation Metric: IoU-Weighted  Score evaluating timing boundary precision and classification accuracy.

### 4.2 ACV Subsystem (Refrigerant Leak Fault Localization)

- Objective: Evaluate multi-car temperature deltas and operational modes to output a ranked car sequence identifying early-stage refrigerant leaks (e.g., 03|01|02|04).

- Algorithms: CatBoost / LightGBM Gradient Boosted Decision Trees (GBDT).

- Mathematical Pipeline:

  - Computes running temperature gradients between cabin ambient  and control targets :

  - Predicts continuous fault likelihood scores  to order the carriages from highest to lowest leak probability.

- Evaluation Metric: Linear Rank-Decay Score giving maximum credit when the true faulty car is placed first:

### 4.3 Rail Corrugation Subsystem (Vibration Multi-Class Classification)

- Objective: Classify axle-box vibration signals into Normal, Side I, or Side II corrugated rail conditions.

- Algorithms: LightGBM Classifier with Time-Domain & Spectral Feature Extraction.

- Mathematical Pipeline:

  - Converts multi-channel accelerometer time-series  to the frequency domain via Fast Fourier Transform (FFT) to derive Spectral Energy Density:

  - Features are classified via Multi-Class Cross-Entropy into three discrete classes.

- Evaluation Metric: Macro  Score:

### 4.4 Structural Health Monitoring Subsystem (Cumulative Fatigue Regression)

- Objective: Predict continuous cumulative fatigue damage () from dynamic stress time-series recordings on bogie frames.

- Algorithms: Physics-Informed Ridge Regression / XGBoost Regressor.

- Mathematical Pipeline:

  - Uses Rainflow-Counting to group stress cycles  at stress ranges . Palmgren-Miner's linear damage accumulation law estimates cumulative damage :

- Evaluation Metric: Maximized Mean Absolute Percentage Error (MAPE) Score:

## 5. Hardware Edge Architecture: SentryRailTrack (SRT) Robot

The hardware unit is a low-profile, track-mounted robotic rover built to traverse underground tunnels and elevated viaduct tracks during non-revenue engineering hours.

- Mechanical Design: Form-factor prototyped in Fusion 360 and Bambu 3D-printed using a low-clearance C-arch aerodynamic shell designed to fit MRT gauge clearances.

- Edge Processing: Powered by an NVIDIA Jetson Orin Nano / Xavier NX board running quantized model binaries (ONNX / TensorRT) inside Docker containers.

- Sensing Suite:

  - Tri-axial axle-box accelerometers and dynamic strain gauges for vibration and stress capture.

  - Top-arch mounted 4K optical camera with high-lumen LED floodlights for overhead catenary and tunnel inspection.

- Offline Resiliency: Underground tunnels lack continuous cellular service; the robot buffers raw telemetry to an industrial onboard NVMe SSD, auto-synchronizing via Wi-Fi 6 mesh when returning to depot range.

## 6. Software Deployment & Competition Compliance

### 6.1 Cloud Dashboard & Serverless API

- Web App Hosting: Hosted live on Vercel (smrtrackbot-ops.vercel.app), providing a non-technical drag-and-drop file interface for .csv, .txt, and .xlsx telemetry.

- Serverless Backend: Implemented using Vercel Native Python Serverless Runtimes executing pre-compiled model weights (.bin, .cbm, .pt) in memory.

- Automated CSV Generation: Generates the four strict, flat CSV outputs (door_predictions.csv, acv_predictions.csv, rail_predictions.csv, shm_predictions.csv) packaged into a root-level predictions.zip archive without subfolders.

### 6.2 Generative AI Explainability (Google AI Studio & Gemini API)

- Integrates the Gemini API using custom system prompts configured in Google AI Studio.

- Converts numerical ML predictions (e.g., an ACV ranking of 03|01|02|04 or an SHM fatigue score) into plain-English diagnostic root-cause summaries and step-by-step SOPs for non-technical maintenance engineers.

### 6.3 Frontline Operator Integration (SRT Mobile App)

- Designed for Senior Rail Technicians (SRT) and frontline response teams.

- Pushes real-time alerts to mobile tablets with high-contrast dark modes, offline-cached tunnel maps, and step-by-step troubleshooting SOPs for field maintenance.

## 7. Future Improvements & Scalability Roadmap

- Physics-Informed Neural Networks (PINNs): Transitioning SHM regression to PINNs that directly incorporate non-linear material stress-strain equations.

- Multimodal Edge Vision: Fine-tuning lightweight vision-language models (e.g., Florence-2 / Gemini Nano) on the Jetson Orin board to correlate optical visual defects with acoustic/vibration signals simultaneously.

- Power Scavenging & Swarm Robotics: Adding passive third-rail induction power harvesting to charge onboard batteries during sweeps, and enabling Sub-GHz mesh coordination between a fleet of rovers across intersecting lines.

- Digital Twin & Enterprise Integration: Incorporating a WebGL/Three.js 3D Digital Twin of the tunnel network into the web dashboard, and linking the Gemini diagnostic layer directly to enterprise CMMS platforms (SAP PM / IBM Maximo) for auto-generating work orders.

## 8. Conclusion & Impact

SentryRailTrack (SRT) transforms rail condition monitoring from manual, reactive checks into an automated, predictive operational paradigm:

| Metric / Dimension | Legacy Operational Baseline | SentryRailTrack (SRT) Solution Impact |

| Inspection Coverage | Fractional coverage per night during 3-hr windows | 100% automated track sweeps across tunnels & viaducts |

| Fault Detection (MTTD) | Hours/Days (caught after physical breakdown) | Instantaneous / Real-time (flagged at micro-degradation) |

| Maintenance Model | Fixed-interval time-based replacement | Condition-based predictive asset management |

| Commuter Experience | Unexpected in-service door jams & cooling trips | Zero peak-hour train withdrawals & reduced track noise |

By unifying edge robotics, dedicated subsystem machine learning models, and explainable LLM diagnostics, SentryRailTrack (SRT) delivers a practical, fully compliant solution that protects rail infrastructure and improves transit reliability across Singapore's MRT network.